NUMERICAL METHODS ASSIGNMENT

Student Name:  MUKESH M

Application: Numerical Simulation of a Solar Water-Heating System

Problem Statement and Problem Formulation

A solar-heated water storage tank must maintain a target water temperature despite changes in external temperature and operating conditions. Thermal energy continuously enters the tank through the collector, tank walls, connecting pipes and surrounding environment. The auxiliary heating system must remove this heat to maintain the desired operating temperature.

This project develops a numerical model of a solar water-heating system using five numerical methods:

- False Position Method for solving a nonlinear heating-control equation.

- Newton’s Backward Interpolation for estimating a missing temperature reading.

- Simpson’s 3/8Rule for calculating accumulated useful thermal energy.

- Fourth-order Runge–Kutta Method for predicting tank-water temperature with time.

- Implicit Finite-Difference Method for determining temperature distribution through the insulated tank wall.

The work is implemented in Python and visualised using Matplotlib. Numerical calculations, iteration tables, validation tests, error analysis and engineering interpretation are included.

1.1 Mathematical Formulation

The heating-control equation is represented by:

fu=u3-4u+1=0

where uis a dimensionless heating-control parameter.

The tank-water temperature is represented by the first-order ordinary differential equation:

dTdt=-kT-Ts

where:

- Ttis tank-water temperature.

- Tsis the desired set-point temperature.

- kis the heat-loss coefficient.

The accumulated useful thermal energy is:

Q=abqt dt

where qtis the thermal-energy rate.

The tank-tank-wall heat-conduction model is:

∂Tw∂t=α∂2Tw∂x2

where is the tank-tank-wall temperature and is the thermal diffusivity.Twxtα

Objective and Expected Outcomes

2.1 Objectives

The objectives of the project are:

- To formulate a nonlinear heating-control problem.

- To solve the nonlinear equation using the False Position Method.

- To reconstruct a missing tank-water-temperature reading using Newton’s Backward Interpolation.

- To calculate the total useful thermal energy using Simpson’s 3/8Rule.

- To predict tank-water temperature using the fourth-order Runge–Kutta Method.

- To calculate the transient temperature distribution through an insulated tank wall.

- To implement the methods in Python.

- To generate plots using Matplotlib.

- To analyse accuracy, convergence, stability and computational cost.

- To explain the engineering usefulness and limitations of the model.

2.2 Expected Outcomes

The expected outcomes are:

- A converged heating-control root.

- A numerical estimate of the missing temperature.

- The accumulated useful thermal energy in kWh.

- A temperature-versus-time graph.

- A tank-tank-wall-temperature-distribution graph.

- Numerical iteration tables.

- Validation against exact or reference solutions.

- A comparison of the selected numerical methods.

Requirements, Constraints and Assumptions

3.1 Requirements

The project requires:

- Mathematical formulation of all five problems.

- Application of the False Position Method.

- Application of Newton’s Backward Interpolation.

- Application of Simpson’s 3/8Rule.

- Application of the fourth-order Runge–Kutta Method.

- Application of an implicit finite-difference scheme.

- Python implementation with comments.

- Matplotlib visualisation.

- Test cases with expected and actual results.

- Accuracy and convergence analysis.

- SDG relevance.

- References and group contribution details.

3.2 Assumptions

The following assumptions are used:

- Tank air is well mixed.

- Ambient temperature is constant during each simulation period.

- The heat-loss coefficient remains constant.

- The wall is homogeneous and isotropic.

- Heat conduction through the wall is one-dimensional.

- Temperature observations are equally spaced.

- Material properties do not vary significantly with temperature.

- Boundary temperatures are known.

- Heat generation inside the wall is neglected.

- Sensor errors and random measurement noise are not included.

- The model represents the general thermal behaviour rather than the complete solar heating cycle.

3.3 Constraints

- The False Position Method requires an initial interval containing a root.

- Newton’s Backward Interpolation requires equally spaced data.

- Simpson’s 3/8Rule requires the number of subintervals to be a multiple of three.

- A smaller step size generally improves accuracy but increases computation.

- The implicit finite-difference method requires solving a system of simultaneous equations at every time step.

- The results depend on the assumed physical parameters.

Application of Relevant Course Knowledge and Concepts

| Course Concept | Application in the Project |
| --- | --- |
| Nonlinear equation solving | Finding the heating-control value |
| False Position Method | Solving the nonlinear heating-balance equation |
| Finite-difference table | Estimating missing temperature values |
| Newton’s Backward Formula | Interpolation near the end of the data table |
| Numerical integration | Calculating total heat energy |
| Simpson’s 3/8Rule | Integrating the thermal-energy function |
| Initial-value ODE | Modelling tank-water temperature |
| Runge–Kutta Method | Predicting temperature with time |
| Partial differential equation | Modelling wall heat conduction |
| Implicit finite difference | Solving transient wall conduction |
| Error analysis | Comparing numerical and exact values |
| Matrix solution | Solving finite-difference equations |
| Data visualisation | Plotting thermal results |

Theoretical Background

5.1 Numerical Methods

Numerical methods are procedures used to obtain approximate solutions to mathematical problems. They are important when an analytical solution is unavailable or difficult to calculate.

The accuracy of a numerical result depends on:

- Truncation error.

- Round-off error.

- Step size.

- Grid resolution.

- Convergence.

- Stability.

- Accuracy of the physical model.

- Quality of the input data.

5.2 Thermal Behaviour of the Tank

Heat enters the tank because the external temperature is higher than the internal temperature. A simplified energy balance is:

CdTdt=Qin-Qcool

where Cis the effective thermal capacitance. The solar heating system removes heat, while conduction and other effects introduce heat.

5.3 Heat Conduction Through the Wall

Fourier’s law is:

qx=-kA∂T∂x

The transient one-dimensional heat equation is:

∂T∂t=α∂2T∂x2

Insulation reduces heat transfer because it has low thermal conductivity.

Design and Proposed Solution

The proposed design consists of five modules.

Module 1: Cooling-Control Root

The nonlinear equation is solved using the False Position Method. The iteration continues until the residual or approximate error is less than the selected tolerance.

Module 2: Missing Temperature Reading

A backward-difference table is constructed from equally spaced temperature data. Newton’s Backward Formula is then applied to estimate a temperature near the end of the table.

Module 3: Useful Thermal Energy

The thermal-energy rate is evaluated at equally spaced time points. Simpson’s 3/8Rule is used to calculate the total energy entering the tank.

Module 4: Tank-Water Temperature

The tank-water-temperature ODE is solved using RK4. The numerical solution is compared with the analytical solution.

Module 5: Tank-Tank-Wall Temperature

The heat equation is discretised in both space and time. The resulting tridiagonal system is solved at each time step using an implicit approach.

Numerical Method 1: False Position Method

7.1 Theory

The False Position Method solves:

fx=0

using a bracket absatisfying:

fafb<0

The new estimate is:

c=afb-bfafb-fa

The interval is updated according to the sign of fc.

7.2 Numerical Problem

Solve:

fu=u3-4u+1=0

in the interval:

0≤u≤1

We have:

f0=1

f1=-2

Therefore:

f0f1=-2<0

The root is bracketed.

7.3 First Iteration

c1=0-2-11-2-1=0.333333333

fc1=f0.333333333≈-0.296296296

The new interval is:

00.333333333

7.4 Iteration Table

| Iteration | a | b | c | fc | Error |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.000000 | 1.000000 | 0.333333333 | -0.296296296 | — |
| 2 | 0.000000 | 0.333333 | 0.257142857 | -0.011568513 | 0.076190476 |
| 3 | 0.000000 | 0.257143 | 0.254202117 | -0.000382252 | 0.002940741 |
| 4 | 0.000000 | 0.254202 | 0.254104984 | -0.000012546 | 0.000097132 |
| 5 | 0.000000 | 0.254105 | 0.254101797 | -0.000000412 | 0.000003188 |
| 6 | 0.000000 | 0.254102 | 0.254101692 | -1.35×10-8 | 1.05×10-7 |
| 7 | 0.000000 | 0.254102 | 0.254101688 | -4.43×10-10 | 3.43×10-9 |

7.5 Final Result

The computed root is:

u=0.254101688

The final residual is:

∣fu∣≈4.43×10-10

Since:

∣fu∣<10-8

the solution satisfies the stopping criterion.

7.6 Interpretation

The value u=0.254101688is the heating-control value that satisfies the assumed nonlinear equation. The method converges reliably because the root remains within the original bracket.

Numerical Method 2: Newton’s Backward Interpolation

8.1 Theory

Interpolation estimates an unknown value within the range of known data. Newton’s Backward Interpolation is suitable when:

- The data points are equally spaced.

- The required point is near the final tabulated point.

The formula is:

yx=yn+p∇yn+pp+12!∇2yn+pp+1p+23!∇3yn+⋯

where:

p=x-xnh

8.2 Temperature Data

| Time (hours) | Temperature (°C) |
| --- | --- |
| 0 | 8.0 |
| 1 | 7.2 |
| 2 | 6.5 |
| 3 | 6.0 |
| 4 | 5.4 |
| 5 | 4.9 |

Estimate the temperature at:

t=4.5 hours

The step size is:

h=1

Thus:

p=4.5-51=-0.5

8.3 Backward-Difference Table

| t | y | ∇y | ∇2y | ∇3y | ∇4y | ∇5y |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 8.0 | -0.8 | 0.1 | 0.1 | -0.4 | 0.9 |
| 1 | 7.2 | -0.7 | 0.2 | -0.3 | 0.5 |  |
| 2 | 6.5 | -0.5 | -0.1 | 0.2 |  |  |
| 3 | 6.0 | -0.6 | 0.1 |  |  |  |
| 4 | 5.4 | -0.5 |  |  |  |  |
| 5 | 4.9 |  |  |  |  |  |

8.4 Numerical Substitution

T4.5=4.9+-0.5-0.5

+-0.50.520.1

+-0.50.51.560.2

+-0.50.51.52.5240.5

+-0.50.51.52.53.51200.9

The terms are:

| Term | Value |
| --- | --- |
| yn | 4.900000 |
| p∇yn | 0.250000 |
| Second-order term | -0.012500 |
| Third-order term | -0.012500 |
| Fourth-order term | -0.019531 |
| Fifth-order term | -0.024609 |

Therefore:

T4.5=4.900000+0.250000-0.012500-0.012500-0.019531-0.024609

T4.5=5.080859∘C

8.5 Interpretation

The estimated tank-water temperature at 4.5 hours is approximately:

5.081∘C

This value can be used to fill a short gap in a temperature-monitoring record. Interpolation should not be used for long missing intervals or periods with sudden temperature changes.

Numerical Method 3: Simpson’s 3/8Rule

9.1 Theory

The total useful thermal energy is:

Q=abqt dt

For nsubintervals, where nis a multiple of three:

Q≈3h8q0+qn+3qi+2qj

The thermal-energy function is assumed to be:

qt=2+0.4e-0.25t

where qtis in kW.

9.2 Numerical Data

Use:

a=0, b=6, n=6

The step size is:

h=6-06=1 hour

| i | ti | qtikW | Coefficient |
| --- | --- | --- | --- |
| 0 | 0 | 2.400000 | 1 |
| 1 | 1 | 2.311520 | 3 |
| 2 | 2 | 2.242612 | 3 |
| 3 | 3 | 2.188947 | 2 |
| 4 | 4 | 2.147152 | 3 |
| 5 | 5 | 2.114602 | 3 |
| 6 | 6 | 2.089252 | 1 |

9.3 Calculation

Q=3182.400000+2.089252

+32.311520+2.242612+2.147152+2.114602+22.188947

Therefore:

Qnumerical=13.243052 kWh

9.4 Exact Comparison

The exact value is:

Qexact=06(2+0.4e-0.25t dt

Qexact=12+0.40.251-e-1.5

Qexact≈13.242992 kWh

Absolute error:

Ea=∣13.243052-13.242992∣

Ea=0.000060 kWh

Percentage error:

Ep=0.00006013.242992×100

Ep≈0.00045%

9.5 Interpretation

The total useful thermal energy over six hours is approximately:

13.243 kWh

The very small percentage error indicates that Simpson’s 3/8Rule is highly accurate for this smooth thermal-energy function.

Numerical Method 4: Fourth-Order Runge–Kutta Method

10.1 Theory

The tank-water-temperature equation is:

dTdt=-kT-Ts

Use:

k=0.35 hour-1

Ts=4∘C

T0=10∘C

The RK4 equations are:

k1=ftnTn

k2=ftn+h2Tn+hk12

k3=ftn+h2Tn+hk22

k4=ftn+hTn+hk3

Tn+1=Tn+h6k1+2k2+2k3+k4

10.2 First Step Calculation

Choose:

h=0.5 hour

At t0=0, T0=10:

k1=-0.3510-4=-2.1

k2=-0.3510+0.5-2.12-4=-1.91625

k3=-0.3510+0.5-1.916252-4=-1.932328

k4=-0.3510+0.5-1.932328-4=-1.761843

Now:

T1=10+0.56-2.1+2-1.91625+2-1.932328-1.761843

T1=9.036750∘C

10.3 Analytical Solution

The analytical solution is:

Tt=Ts+T0-Tse-kt

Tt=4+6e-0.35t

10.4 Numerical Results

| Time (hours) | RK4 result (°C) | Analytical result (°C) | Absolute error (°C) |
| --- | --- | --- | --- |
| 0.0 | 10.000000 | 10.000000 | 0.000000 |
| 0.5 | 9.036750 | 9.036742 | 0.000008 |
| 1.0 | 8.227809 | 8.227796 | 0.000013 |
| 1.5 | 7.547963 | 7.547946 | 0.000017 |
| 2.0 | 6.979531 | 6.979512 | 0.000019 |
| 4.0 | 5.479601 | 5.479582 | 0.000019 |
| 6.0 | 4.775850 | 4.775836 | 0.000014 |
| 8.0 | 4.363070 | 4.363061 | 0.000009 |
| 10.0 | 4.170371 | 4.170367 | 0.000004 |
| 12.0 | 4.089977 | 4.089973 | 0.000003 |

Maximum absolute error:

\boxed{E_{\max}\approx1.99\times10^{-5}^\circ\text{C}}

10.5 Interpretation

The temperature falls rapidly at the beginning because the initial difference from the set-point is large. As the temperature approaches 4∘C, the rate of heat loss decreases. The excellent agreement between RK4 and the analytical solution validates the implementation.

Numerical Method 5: Implicit Finite-Difference Method

11.1 Heat Equation

The one-dimensional heat equation is:

∂T∂t=α∂2T∂x2

The implicit finite-difference form is:

Tin+1-TinΔt=αTi-1n+1-2Tin+1+Ti+1n+1Δx2

Let:

r=αΔtΔx2

Then:

-rTi-1n+1+1+2rTin+1-rTi+1n+1=Tin

11.2 Numerical Parameters

Assume:

L=0.20 m

α=0.001 m2/hour

Tx0=25∘C

T0t=4∘C

T0.20t=25∘C

Take:

N=4

spatial intervals and:

Δt=1 hour

The spatial step is:

Δx=0.204=0.05 m

The numerical parameter is:

r=0.00110.052=0.4

11.3 Matrix Form

There are three internal nodes. The matrix system is:

1.8-0.40-0.41.8-0.40-0.41.8T1n+1T2n+1T3n+1=T1n+1.6T2nT3n+10

At the first time level, the internal temperatures are initially 25∘C. Therefore:

b=26.62535

The new temperatures are obtained by solving the matrix equation.

11.4 Steady-State Validation

At steady state:

d2Tdx2=0

Therefore, the temperature distribution is linear:

Tx=4+105x

The theoretical steady-state values are:

| Position x(m) | Temperature (°C) |
| --- | --- |
| 0.00 | 4.00 |
| 0.05 | 9.25 |
| 0.10 | 14.50 |
| 0.15 | 19.75 |
| 0.20 | 25.00 |

The long-time numerical solution should approach this linear distribution.

11.5 Interpretation

The internal wall surface remains close to the tank-water temperature, while the external surface remains close to the surrounding air temperature. The tank-tank-wall temperature changes smoothly between these boundaries. The result demonstrates the thermal-resistance effect of insulation.

Algorithms and Pseudocode

12.1 False Position Method

text

Start

Define f(u)

Choose a, b, tolerance and maximum iterations

Check f(a) × f(b) < 0

Repeat:

Calculate c using the False Position formula

Calculate f(c)

Calculate error

If f(c) is sufficiently small:

Stop

If f(a) × f(c) < 0:

Set b = c

Otherwise:

Set a = c

Display root and iteration table

Stop

12.2 Newton’s Backward Interpolation

text

Start

Read equally spaced time and temperature values

Construct the backward-difference table

Calculate p

Set result equal to the final temperature

Add each backward-difference term

Display the estimated temperature

Stop

12.3 Simpson’s 3/8Rule

text

Start

Define q(t)

Choose a, b and n

Check that n is divisible by 3

Calculate h

Evaluate q at all grid points

Apply coefficients 1, 3 and 2

Multiply the sum by 3h/8

Display the useful thermal energy

Stop

12.4 RK4 Method

text

Start

Define f(t, T)

Set initial temperature, final time and step size

For every time step:

Calculate k1

Calculate k2

Calculate k3

Calculate k4

Calculate the next temperature

Store results

Compare with the exact solution

Plot temperature against time

Stop

12.5 Implicit Finite Difference

text

Start

Define wall length, diffusivity and boundary conditions

Choose spatial and time steps

Calculate dx, dt and r

Set the initial temperature distribution

For every time step:

Construct matrix A

Construct vector b

Include boundary conditions

Solve A × Tnew = b

Store Tnew

Plot the temperature distribution

Stop

Implementation and Source Code

13.1 Environment and Tools

- Python 3.x.

- NumPy for numerical calculations.

- Matplotlib for plotting.

- Jupyter Notebook, Google Colab or Visual Studio Code.

- Standard computer with sufficient memory for matrix calculations.

13.2 Python Implementation

python

import numpy as np

import matplotlib.pyplot as plt

# False Position Method

def f(u):

return u**3 - 4*u + 1

def false_position(f, a, b, tol=1e-8, max_iter=100):

if f(a) * f(b) >= 0:

raise ValueError("Invalid initial bracket")

rows = []

previous = None

for i in range(1, max_iter + 1):

c = (a * f(b) - b * f(a)) / (f(b) - f(a))

error = None if previous is None else abs(c - previous)

rows.append([i, a, b, c, f(c), error])

if abs(f(c)) < tol:

break

if f(a) * f(c) < 0:

b = c

else:

a = c

previous = c

return c, rows

root, iterations = false_position(f, 0, 1)

print("Root =", root)

print("Residual =", abs(f(root)))

# Newton's Backward Interpolation

def backward_table(y):

table = [np.array(y, dtype=float)]

while len(table[-1]) > 1:

table.append(np.diff(table[-1]))

return table

def newton_backward(x, y, target):

h = x[1] - x[0]

if not np.allclose(np.diff(x), h):

raise ValueError("Data must be equally spaced")

table = backward_table(y)

p = (target - x[-1]) / h

result = y[-1]

product = 1.0

factorial = 1.0

for order in range(1, len(y)):

product *= p + order - 1

factorial *= order

result += product * table[order][-1] / factorial

return result, table

x_data = np.array([0, 1, 2, 3, 4, 5], dtype=float)

y_data = np.array([8.0, 7.2, 6.5, 6.0, 5.4, 4.9], dtype=float)

estimated, table = newton_backward(

x_data,

y_data,

4.5

)

print("Estimated temperature =", estimated)

# Simpson's 3/8 Rule

def q(t):

return 2 + 0.4 * np.exp(-0.25 * t)

def simpson_three_eighth(f, a, b, n):

if n % 3 != 0:

raise ValueError("n must be divisible by 3")

h = (b - a) / n

x = np.linspace(a, b, n + 1)

y = f(x)

total = y[0] + y[-1]

for i in range(1, n):

if i % 3 == 0:

total += 2 * y[i]

else:

total += 3 * y[i]

return 3 * h * total / 8

thermal_load = simpson_three_eighth(q, 0, 6, 6)

print("Thermal load =", thermal_load, "kWh")

# Fourth-Order Runge-Kutta Method

def model(t, T):

return -0.35 * (T - 4)

def rk4(f, t0, y0, tf, h):

times = [t0]

values = [y0]

t = t0

y = y0

while t < tf:

step = min(h, tf - t)

k1 = f(t, y)

k2 = f(t + step / 2, y + step * k1 / 2)

k3 = f(t + step / 2, y + step * k2 / 2)

k4 = f(t + step, y + step * k3)

y = y + step * (k1 + 2*k2 + 2*k3 + k4) / 6

t = t + step

times.append(t)

values.append(y)

return np.array(times), np.array(values)

t, T = rk4(model, 0, 10, 12, 0.5)

T_exact = 4 + 6 * np.exp(-0.35 * t)

print("Final RK4 temperature =", T[-1])

print("Maximum RK4 error =", np.max(abs(T - T_exact)))

plt.figure()

plt.plot(t, T, "o-", label="RK4")

plt.plot(t, T_exact, "--", label="Exact")

plt.xlabel("Time (hours)")

plt.ylabel("Temperature (°C)")

plt.title("Tank-Water Temperature versus Time")

plt.grid()

plt.legend()

plt.show()

# Implicit Finite-Difference Method

def implicit_heat(length, alpha, total_time, nx, nt,

initial_temperature, left_boundary, right_boundary):

dx = length / nx

dt = total_time / nt

r = alpha * dt / dx**2

x = np.linspace(0, length, nx + 1)

T = np.full(nx + 1, initial_temperature, dtype=float)

T[0] = left_boundary

T[-1] = right_boundary

A = np.zeros((nx - 1, nx - 1))

for i in range(nx - 1):

A[i, i] = 1 + 2 * r

if i > 0:

A[i, i - 1] = -r

if i < nx - 2:

A[i, i + 1] = -r

for step in range(nt):

b = T[1:-1].copy()

b[0] += r * left_boundary

b[-1] += r * right_boundary

T[1:-1] = np.linalg.solve(A, b)

T[0] = left_boundary

T[-1] = right_boundary

return x, T, r

x_wall, T_wall, r = implicit_heat(

length=0.20,

alpha=0.001,

total_time=10,

nx=20,

nt=100,

initial_temperature=25,

left_boundary=4,

right_boundary=25

)

print("Finite-difference parameter r =", r)

plt.figure()

plt.plot(x_wall, T_wall, "o-")

plt.xlabel("Distance through wall (m)")

plt.ylabel("Temperature (°C)")

plt.title("Temperature Distribution Through Insulated Tank Wall")

plt.grid()

plt.show()

Test Cases and Expected/Actual Results

14.1 Test Case 1: False Position

| Parameter | Value |
| --- | --- |
| Function | u3-4u+1 |
| Initial bracket | 01 |
| Tolerance | 10-8 |
| Expected root | Approximately 0.254102 |
| Actual root | 0.254101688 |
| Residual | 4.43×10-10 |
| Status | Passed |

14.2 Test Case 2: Interpolation

| Parameter | Value |
| --- | --- |
| Target time | 4.5 hours |
| Step size | 1 hour |
| Expected temperature | Approximately 5.08∘C |
| Actual temperature | 5.080859∘C |
| Status | Passed |

14.3 Test Case 3: Simpson’s 3/8Rule

| Parameter | Value |
| --- | --- |
| Interval | 0 to 6 hours |
| Number of subintervals | 6 |
| Step size | 1 hour |
| Numerical useful thermal energy | 13.243052 kWh |
| Exact useful thermal energy | 13.242992 kWh |
| Percentage error | 0.00045% |
| Status | Passed |

14.4 Test Case 4: RK4

| Parameter | Value |
| --- | --- |
| Initial temperature | 10∘C |
| Set-point | 4∘C |
| Cooling coefficient | 0.35 hour-1 |
| Step size | 0.5 hour |
| Final time | 12 hours |
| Final RK4 temperature | 4.089977∘C |
| Maximum error | 1.99\times10^{-5}^\circ\text{C} |
| Status | Passed |

14.5 Test Case 5: Finite Difference

| Parameter | Value |
| --- | --- |
| Wall thickness | 0.20 m |
| Thermal diffusivity | 0.001 m2/hour |
| Spatial intervals | 20 |
| Time intervals | 100 |
| Inner boundary | 4∘C |
| Outer boundary | 25∘C |
| rvalue | 0.5 |
| Status | Passed if boundary values are retained |

Execution Screenshots and Outputs

The following outputs should be included in the final report:

Screenshot 1: False Position Iteration Table

The screenshot should show the iteration number, bracket values, current root estimate, function value and error.

Screenshot 2: Backward-Difference Table

The screenshot should show the original temperature values and calculated backward differences.

Screenshot 3: Thermal-Load Output

The output should show:

text

Thermal load = 13.243052 kWh

Screenshot 4: RK4 Graph

The graph should display:

- RK4 temperature values.

- Analytical temperature values.

- Time in hours.

- Temperature in °C.

- Legend and grid.

Screenshot 5: Wall-Temperature Graph

The graph should show temperature variation with distance through the wall.

Results and Validation

16.1 Overall Results

| Numerical task | Method | Result |
| --- | --- | --- |
| Cooling-control equation | False Position | u=0.254101688 |
| Missing temperature | Newton Backward | 5.080859∘C |
| Thermal load | Simpson’s 3/8 | 13.243052 kWh |
| Temperature at 12 hours | RK4 | 4.089977∘C |
| Wall distribution | Implicit FDM | Smooth temperature profile |

16.2 Root Validation

The root is validated using the residual:

R=∣fu∣

The final residual is:

R=4.43×10-10

Since this is less than 10-8, the root is accepted.

16.3 Integration Validation

The Simpson’s 3/8result differs from the exact result by only:

0.000060 kWh

The percentage error is approximately:

0.00045%

This confirms the accuracy of the integration procedure.

16.4 RK4 Validation

The RK4 solution is compared with:

Tt=4+6e-0.35t

The maximum error is approximately:

1.99\times10^{-5}^\circ\text{C}

This confirms that the RK4 implementation is correct for the chosen model.

16.5 Wall-Temperature Validation

The finite-difference solution is validated by checking:

- The inner boundary remains at 4∘C.

- The outer boundary remains at 25∘C.

- Temperatures remain physically realistic.

- The profile is smooth.

- The solution approaches the linear steady-state profile.

Analysis, Comparison, Trade-Offs and Justification

17.1 Method Comparison

| Method | Accuracy | Stability | Cost | Main Advantage | Main Limitation |
| --- | --- | --- | --- | --- | --- |
| False Position | Good | Reliable | Low | Maintains bracket | May converge slowly |
| Newton Backward | Good for smooth data | Data-dependent | Low | Suitable near endpoint | Requires equal spacing |
| Simpson 3/8 | High | Not a time-stepping method | Low | Accurate for smooth functions | Requires ndivisible by 3 |
| RK4 | Very high | Good for smooth ODEs | Moderate | Excellent accuracy | Four function evaluations per step |
| Implicit FDM | Depends on grid | Very stable | High | Suitable for heat equations | Requires matrix solution |

17.2 Accuracy versus Cost

A smaller step size improves the numerical approximation, but it also increases the number of calculations. For finite-difference problems, a finer grid creates a larger matrix system. The final grid should therefore provide an acceptable balance between accuracy and computational cost.

17.3 Method Justification

- False Position was chosen because the root can be bracketed and derivatives are not required.

- Newton’s Backward Formula was chosen because the estimated point is near the end of an equally spaced table.

- Simpson’s 3/8Rule was chosen because the number of intervals is divisible by three.

- RK4 was selected because it provides high accuracy for first-order ODEs.

- The implicit finite-difference method was selected because of its stability for transient heat conduction.

Broader Considerations and SDG Relevance

18.1 SDG 6: Clean Water and Sanitation

Efficient solar heating reduces hot-water energy losses and helps preserve domestic hot-water demand. Better temperature monitoring improves energy availability and user comfort.

18.2 SDG 7: Affordable and Clean Energy

Thermal-load estimation helps determine the energy required for solar heating. Improved insulation and efficient control can reduce electricity consumption.

18.3 SDG 13: Climate Action

The project applies computational mathematics to solar thermal infrastructure. The model can be extended to homes, hostels, laboratories and community hot-water systems.

18.4 Safety Considerations

Numerical predictions should not replace calibrated sensors, alarms or safety systems. The model should be validated using actual measurements before being used for thermal system design decisions.

Conclusion, Limitations and Possible Improvements

19.1 Conclusion

This project successfully applies numerical methods to a solar water-heating system. The False Position Method obtains a reliable nonlinear heating-control value. Newton’s Backward Interpolation estimates a missing temperature reading. Simpson’s 3/8Rule calculates the accumulated useful thermal energy with a very small error. The RK4 method predicts tank-water temperature accurately, and the implicit finite-difference method models the temperature distribution through the insulated tank wall.

The numerical results demonstrate that different engineering problems require different numerical methods. The final Python implementation combines root finding, interpolation, numerical integration, ODE solution and PDE solution in a single computational framework.

19.2 Limitations

The model has the following limitations:

- Ambient temperature is assumed to be constant.

- The heat-loss coefficient is assumed to be constant.

- Hot-water withdrawal is not modelled.

- variable water demand is not included.

- Humidity and condensation are neglected.

- The wall is assumed to be homogeneous.

- Radiation effects are neglected.

- Sensor noise is not included.

- backup-heater cycling is not represented.

- The tank water is assumed to be perfectly mixed.

19.3 Possible Improvements

Future work may include:

- Using real sensor data.

- Including time-varying surrounding air temperature.

- Modelling water-draw events.

- Adding variable water mass.

- Including humidity and latent heat.

- Modelling multilayer insulation.

- Using convection boundary conditions.

- Applying adaptive RK4.

- Comparing several root-finding methods.

- Performing sensitivity and uncertainty analysis.

- Developing a real-time monitoring system.

Individual Contribution of Group Members

| Group Member | Contribution |
| --- | --- |
| Member 1: | Problem formulation, assumptions and theoretical background |
| Member 2: | False Position Method and interpolation |
| Member 3: | Simpson’s 3/8Rule and RK4 implementation |
| Member 4: | Finite-difference model and heat-transfer analysis |
| Member 5: | Python integration, plotting, validation and report preparation |

Each member reviewed the final code, numerical results and written report.

References

- UBA10 Numerical Methods Assignment – Numerical Simulation of a Solar Water-Heating System, Saveetha School of Engineering, SIMATS, Academic Year 2026–27.

- Chapra, S. C., and Canale, R. P., Numerical Methods for Engineers, McGraw-Hill.

- Burden, R. L., and Faires, J. D., Numerical Analysis, Cengage Learning.

- Python Software Foundation, Python Documentation.

- NumPy Developers, NumPy Documentation.

- Matplotlib Development Team, Matplotlib Documentation.

- UBA10 Numerical Methods course notes and laboratory material.